TEST CASES

PSU Control

ALBERTO IBARRONDO 18/03/2015

INDEX

INTRODUCTION	3

INTERFACE	4

HARDWARE	4

MTTTY INTERFACE	4

RDAC – AD5292	4

NIELSEN HEURISTICS FOR WINDOWS INTERFACE	5

FUNCTIONAL	5

CONFIGURATION	5

RDAC_COMPLETE	5

RDAC_CTRL	6

RDAC_CONFIG	7

RDAC_FEEDBACK	8

FLASH_MEM	9

COMMUNICATION	10

I2C Bus	10

SPI BUS	11

PSU Control - TESt CASES

# INTRODUCTION

This document fully describes every test needed to ensure correct system functioning. Most tests work under a “black box” philosophy, analyzing the outputs to a certain input, without further inspection. The test cases documentation follows a rigid structure (explanation goes in brackets):

| TC_I_345 (Test Case ID Number, unique for each) | TC_I_SOFTWARE_USER_INTERFACE_START_PROCESS (Test Case ID Name, which inherits the name from the System Requirement it belongs to {in this case it would be SR_I_SOFTWARE_USER_INTERFACE} and adds a unique ending to it) |
| --- | --- |
| Belongs to | SR_I_SOFTWARE_USER_INTERFACE (System Requirement the test links to). |
| Description | Brief explanation of the test, including its LINK TO THE PROGRAMMED TEST CODE in Test_PSU.cpp, usually within a specific method. |
| Required | Hadrware & Software status at the beginning of the test |
| Input | Data do be introduced to the test. Includes the KEYBOARD SHORTCUT to be used in the Test Menu for initialization. |
| Output | Data to receive from the test &interpretation of the data. PASSED: if the output satisfies several conditions NOT PASSED: otherwise |
| RESULTS – Date when the test was passed | Successful/unsuccessful and a proof of it (screenshots or pictures) |

All the software-based tests are stored in the same file: Test_PSU.cpp (with its associated header file Test_CPU.h). Hardware testing is also detailed here.

All of the software testing is called inserting the TestMain() function into the program’s main loop.

In addition to all the test documentation, a troubleshooting guide will be included for each common error found. The following template will accommodate all this info for each epigraph:

| “ERROR 31: Create File” |  |
| --- | --- |
| Description | An explanation why this error occurred |
| Solution | How to solve the error |

# INTERFACE

## HARDWARE

### MTTTY INTERFACE

| “ERROR 2: Create File” |  |
| --- | --- |
| Description | The MOD5213 isn’t communicating with the PC. |
| Solution | Check the selected port (COM5 normally used) and the USB connection. If problem persists, extract the MOD5213 from the test board & insert it again. |

### RDAC – AD5292

| TC_I_001 | TC_I_HARDWARE_RDAC |
| --- | --- |
| Belongs to | SR_I_HARDWARE |
| Description | Checking with the different software methods that the output voltage varies in the desired ranges. No code available. |
| Required | MOD5213 connected to a single SlotCard with I2C bus operational. The SlotCard must be connected to the System Supply. A Voltmeter to monitor the output voltage |
| Input | No keyboard shortcut. Requires a changing value for the RDAC Value Register using a loop to cover a desired interval. |
| Output | The values registered in the voltmeter. In the results table, the values are: Vcalc = 1,25*(20.000*(1-RP/1023)/800) – Expected Value Vout = measurement results – Real Value PASSED: Values follow the mathematic expression. Small relative error NOT PASSED: Don’t. Big relative error |
| 03/03/15 | SUCCESSFUL – Data stored in PSUControl/Tests/ TC_I_HARDWARE_RDAC – Results.xlsx |

## software

| TRAP in MTTTY |  |
| --- | --- |
| Description | When initializing, the MOD5213 doesn’t work, printing a TRAP everytime |
| Solution | The use of printf seems to cause big trouble. The only reason for using it would be printing a FLOAT value, which now can be done with string ftos(float). |

| OSTask doesn’t Work |  |
| --- | --- |
| Description | When using a Task as well as PIT0, the Task doesn’t execute |
| Solution | This is due to the use of PIT0 as a timer interruption, as it’s being used for OSDelay functions. Use PIT1 instead |

## COMMUNICATION

### I2C Bus

| TC_I_002 | TC_I_COMMUNICATION_I2C |
| --- | --- |
| Belongs to | SR_I_ COMMUNICATION |
| Description | Checking the I2C bus with the oscilloscope while transmitting any message. No special code required: any I2C test from TC_F_COMMUNICATION_I2C will suffice (Best used with TC_F_COMMUNICATION_I2C_SIMPLE) |
| Required | MOD5213 connected to a single SlotCard with I2C bus operational. The SlotCard must be connected to the System Supply. An oscilloscope with two probes: one for I2C clock bus and the other for I2C Data bus. Trigger set to rising flank in the I2C Clock Bus, with a level far superior to noise. |
| Input | Calling any i2C function. Best used with “w” |
| Output | Data seen in I2C Data Bus trace. PASSED: Data seen in I2C Data Bus trace matches the theoretical output. NOT PASSED: Doesn’t match. |
| 01/03/15 | SUCCESSFUL – No results stored |

### SPI Bus

| TC_I_003 | TC_I_COMMUNICATION_SPI |
| --- | --- |
| Belongs to | SR_I_ COMMUNICATION |
| Description | Checking the SPI bus with the oscilloscope while transmitting any message. No special code required: any RDAC test from TC_F_CONFIGURATION_RDAC will suffice. |
| Required | MOD5213 connected to a single SlotCard with I2C bus operational. The SlotCard must be connected to the System Supply. An oscilloscope with two probes: one for SPI clock bus and the other for SPI MISO Data bus. Trigger set to rising flank in the SPI Clock Bus, with a level far inferior to noise. |
| Input | Calling any SPI function (e.g. RDAC tests) |
| Output | Data seen in SPI MISO Data bus trace. PASSED: Data seen SPI MISO Data bus trace matches the theoretical output (described in RDACLibrary.cpp epigraph BUFFER METHODS NOT PASSED: Doesn’t match. |
| 01/03/15 | SUCCESSFUL – No results stored |

## NIELSEN HEURISTICS FOR WINDOWS INTERFACE

---PENDING---

# FUNCTIONAL

## MONITORING

| TC_F_001 | TC_F_MONITORING_SCAN_VOLTAGE_SINGLE |
| --- | --- |
| Belongs to | SR_F_MONITORING_SCAN_VOLTAGE |
| Description | Reads a single value from the ADC pin and scales it to voltage. |
| Required | MOD5213 connected to a single SlotCard with the ADC pin connected. The SlotCard must be connected to the System Supply. |
| Input | “z” – Keyboard shortcut from the Test Menu. |
| Output | A voltage Value PASSED: Console-printed voltage matches to the input voltage NOT PASSED: Doesn’t match |
|  | NOT YET DEFINED |

| TC_F_002 | TC_F_MONITORING_SCAN_CURRENT_SINGLE |
| --- | --- |
| Belongs to | SR_F_MONITORING_SCAN_CURRENT |
| Description | Reads a single value from the ADC pin and scales it to current. |
| Required | MOD5213 connected to a single SlotCard with the ADC pin connected. The SlotCard must be connected to the System Supply. |
| Input | “x” – Keyboard shortcut from the Test Menu. |
| Output | A current Value PASSED: Console-printed current matches to the input current NOT PASSED: Doesn’t match |
|  | NOT YET DEFINED |

## CONFIGURATION

### RDAC_COMPLETE

| TC_F_200 | TC_F_CONFIGURATION_RDAC_BATTERY_TEST |
| --- | --- |
| Belongs to | SR_F_CONFIGURATION_RDAC_CTRL SR_F_CONFIGURATION_RDAC_CONFIG SR_F_CONFIGURATION_RDAC_FEEDBACK |
| Description | Battery of tests to check all the RDAC functions (RDACLibrary.cpp). Includes the option to measure output values when a change is executed. Code is in function BATTERY_TEST_RDAC() in Test_PSU.cpp |
| Required | MOD5213 connected to a single SlotCard with I2C bus operational. The SlotCard must be connected to the System Supply. |
| Input | “u” – Keyboard shortcut from the Test Menu. “1”/”2” – Select the slave to be tested (Upper o Lower). “y”/”n” – Check the output voltage during the test (with a voltmeter) A new value for the output voltage. |
| Output | A detailed list of the results for each function. PASSED: Self-explanatory NOT PASSED: Self-explanatory |
| 13/03/15 | SUCCESSFUL – Complete Screenshots in PSUControl/Tests/TC_F_CONFIGURATION_RDAC – Results.docx |

### RDAC_CTRL

| TC_F_201 | TC_F_CONFIGURATION_RDAC_CTRL_SIMPLE |
| --- | --- |
| Belongs to | SR_F_CONFIGURATION_RDAC_CTRL |
| Description | Test for setCtrlRDAC() function, sending a “1” to allow RDAC value updating, then a “0” to prevent it (alternatively) |
| Required | MOD5213 connected to a single SlotCard with I2C bus operational. The SlotCard must be connected to the System Supply. |
| Input | “3” – Keyboard shortcut from the Test Menu. |
| Output | NONE – Should be used with Keyboard shortcut “4” to check for changes. In this case: PASSED: “Register programming” switches from “Disabled” to “Enabled” in the console output. A 2 is received in the last 4 bits of the “Data Received” when “Enabled”. A 0 is received in the last 4 bits of the “Data Received” when “Disabled” NOT PASSED: Doesn’t change. |
| 11/03/15 | SUCCESSFUL – Complete Screenshots in PSUControl/Tests/TC_F_CONFIGURATION_RDAC – Results.docx |

### RDAC_VALUE

| TC_F_202 | TC_F_CONFIGURATION_RDAC_VALUE_SIMPLE |
| --- | --- |
| Belongs to | SR_F_CONFIGURATION_RDAC_VALUE |
| Description | Test for setValRDAC() function, sending a new value from 0 to 1023 to the RDAC. It also uses the scanFloatValue() function to scan the desired value in Volts (In a range from 1.25 to 32), prompting the user to write a value. |
| Required | MOD5213 connected to a single SlotCard with I2C bus operational. The SlotCard must be connected to the System Supply. |
| Input | “1” – Keyboard shortcut from the Test Menu. A new value in volts when asked. |
| Output | NONE – Should be used with Keyboard shortcut “2” to check for changes, or with a voltmeter to monitor the output voltage. In this case: PASSED: Values change. Data Received = Hex value we programmed (840=0x348) NOT PASSED: Don’t change. |
| 11/03/15 | SUCCESSFUL – Complete Screenshots in PSUControl/Tests/TC_F_CONFIGURATION_RDAC – Results.docx |

| TC_F_203 | TC_F_CONFIGURATION_RDAC_VALUE_RESET |
| --- | --- |
| Belongs to | SR_F_CONFIGURATION_RDAC_ VALUE |
| Description | Test for resetRDAC() function, setting the value 0x200 (midscale, 16V) to the RDAC. |
| Required | MOD5213 connected to a single SlotCard with I2C bus operational. The SlotCard must be connected to the System Supply. |
| Input | “5” – Keyboard shortcut from the Test Menu |
| Output | NONE – Should be used with Keyboard shortcut “2” to check for changes, or with a voltmeter to monitor the output voltage. In this case: PASSED: Voltage changes to ~16.8V. Data Received = 0x200 NOT PASSED: Other. |
| 11/03/15 | SUCCESSFUL – Complete Screenshots in PSUControl/Tests/TC_F_CONFIGURATION_RDAC – Results.docx Voltage output was checked with a voltmeter to be 16.8 V |

| Values don’t change |  |
| --- | --- |
| Description | The RDAC values don’t change, but the SPI connection is working. |
| Solution | ¿Have you configured Control Register to allow value updating? Do so. If it still doesn’t work, try a different set of SPI channel configuration values, lowering the frequency (the RDAC might not be reading the orders due to a way-too-fast SPI channel) If your value is always 0, check the upcoming section: RDAC_FEEDBACK |

### RDAC_FEEDBACK

| TC_F_204 | TC_F_CONFIGURATION_RDAC_FEEDBACK_CONTROL |
| --- | --- |
| Belongs to | SR_F_CONFIGURATION_RDAC_FEEDBACK |
| Description | Checking the getCtrlRDAC() function, reading the Control Register from the RDAC. |
| Required | MOD5213 connected to a single SlotCard with I2C bus operational. The SlotCard must be connected to the System Supply. |
| Input | “4” – Keyboard shortcut from the Test Menu |
| Output | PASSED: “Register programming” switches from “Disabled” to “Enabled” in the console output. A 2 is received in the last 4 bits of the “Data Received” when “Enabled”. A 0 is received in the last 4 bits of the “Data Received” when “Disabled”. Best used with command “3” to guarantee proper working. NOT PASSED: Other Values. Everything set to 0. |
| 11/03/15 | SUCCESSFUL – Complete Screenshots in PSUControl/Tests/TC_F_CONFIGURATION_RDAC – Results.docx |

| Received Values are always 0 |  |
| --- | --- |
| Description | The RDAC values change, but no answer is received |
| Solution | The MISO channel isn’t working properly. Check if there is a pull up resistor (2K2), as there should be one. ¿Have you set the other slave to HIGH IMPEDANCE with the function highImpRDAC()? To receive an answer from one slave, the other must be set in high impedance mode. |

| TC_F_205 | TC_F_CONFIGURATION_RDAC_FEEDBACK_VALUE |
| --- | --- |
| Belongs to | SR_F_CONFIGURATION_RDAC_FEEDBACK |
| Description | Checking the getValRDAC() function, reading the Value Register from the RDAC. Best used by setting a concrete value first (or resetting to value 0x200). |
| Required | MOD5213 connected to a single SlotCard with I2C bus operational. The SlotCard must be connected to the System Supply. |
| Input | “2” – Keyboard shortcut from the Test Menu. Best used with previous command “1” |
| Output | PASSED: Value matches the programmed one. NOT PASSED: Other Values. Everything set to 0. |
| 11/03/15 | SUCCESSFUL – Complete Screenshots in PSUControl/Tests/TC_F_CONFIGURATION_RDAC – Results.docx |

| TC_F_206 | TC_F_CONFIGURATION_RDAC_20TP_PROGRAMMING |
| --- | --- |
| Belongs to |  |
| Description |  |
| Required | MOD5213 connected to a single SlotCard with I2C bus operational. The SlotCard must be connected to the System Supply. |
| Input |  |
| Output |  |
| 27/04/15 | SUCCESSFUL – Complete Screenshots in PSUControl/Tests/TC_F_CONFIGURATION_RDAC – Results.docx |

### PSU_TYPE

| SR_F_206 TC_F_CONFIGURATION_PSU_TYPE_PRINT |  |
| --- | --- |
| Belongs to | SR_ F_CONFIGURATION_PSU_TYPE |
| Description | Test for the struct PSU_TYPE that will be used in the rest of the system. Prints all the values for a PSU. No code defined. |
| Required | MOD5213. |
| Input | - |
| Output | Console output PASSED: Complete & functional structure. Values change; objects can be accessed, as well as their parameters. NOT PASSED: Otherwise |
| 24/02/15 | SUCCESSFUL – Complete Screenshots in PSUControl/Tests/ TC_F_CONFIGURATION_PSU_TYPE - Results |

### FLASH_MEM

| SR_F_207 TC_F_CONFIGURATION_FLASH_MEM_BATTERY |  |
| --- | --- |
| Belongs to | SR_F_CONFIGURATION_FLASH_MEM |
| Description | Test for flash memory saving & recovering functions from PSULibrary.cpp (LoadFlashValuesPSU, saveInFlashValuesPSU, readFlashValuesPSU). Also checks the auxiliary methods to set default values and print values for PSU_TYPE objects. (defaultValuesPSU, printValuesPSU). Battery test implemented in BATTERY_TEST_FLASH_MEM (). |
| Required | MOD5213. |
| Input | “y” – Keyboard shortcut from the Test Menu |
| Output | A detailed list of results for all functionalities. PASSED: Self-explanatory NOT PASSED: Self-explanatory |
| 24/03/15 | SUCCESSFUL – Complete Screenshots in PSUControl/Tests/TC_F_CONFIGURATION_RDAC – Results.docx |

| Printf doesn’t print values |  |
| --- | --- |
| Description | When invoked, output method printf() displays correctly all the text, but generates a trap when sending a value. It might be due to a failure in the FIFO piles of the MCF. |
| Solution | Use the newly created function ftos() to convert a float to string, and print it with iprintf() in a %s parameter. |

## COMMUNICATION

### I2C Bus

I2C bus communication can be indirectly checked by running any of the RDAC tests (as they require I2C communication to work). Some simple tests for I2C have been defined below, as well as I2C addressing test for I2CtoSPIBridge.

| SR_F_007 TC_F_COMMUNICATION_I2C_SIMPLE |  |
| --- | --- |
| Belongs to | TC_F_COMMUNICATION_I2C |
| Description | Test for I2C communication, selecting a I2C address destination and sending a short message that is prompted by the user. |
| Required | MOD5213 connected to a single SlotCard with I2C bus operational. |
| Input | “w” – Keyboard shortcut from the Test Menu |
| Output | The result of I2C communication PASSED: message “I2C OK” NOT PASSED: message “I2C ERROR [1-9]” |
|  | NOT PASSED |

| SR_F_301 TC_F_COMMUNICATION_I2C_SPI_CONFIG |  |
| --- | --- |
| Belongs to | TC_F_COMMUNICATION_I2C |
| Description | Shortest test for I2C communication, sending an I2C predefined message. |
| Required | MOD5213 connected to a single SlotCard with I2C bus operational. |
| Input | “q” – Keyboard shortcut from the Test Menu |
| Output | The result of I2C communication PASSED: message “I2C OK” NOT PASSED: message “I2C ERROR [1-9]” |
| 24/03/15 | SUCCESSFUL – Complete Screenshots in PSUControl/Tests/TC_F_COMMUNICATION_I2C – Results.docx |

| SR_F_302 TC_F_COMMUNICATION_I2C_ADDRESSING |  |
| --- | --- |
| Belongs to | TC_F_COMMUNICATION_I2C |
| Description | Test for I2C communication with the I2CtoSPIBridge, changing its address from 0x28 to 0x2F using the 3 switches and rebooting the system. Battery test implemented in BATTERY_TEST_I2C (). |
| Required | MOD5213 connected to a single SlotCard with I2C bus operational. The SlotCard must be connected to the System Supply. |
| Input | “t” – Keyboard shortcut from the Test Menu |
| Output | PASSED: Self-explanatory NOT PASSED: Self-explanatory |
| 24/03/15 | SUCCESSFUL – Complete Screenshots in PSUControl/Tests/TC_F_COMMUNICATION_I2C – Results.docx |

### SPI BUS

Any RDAC function can be used to check for SPI communication. SPI bus works if the “Receive Buffer” is not empty.

| SR_F_303 TC_F_COMMUNICATION_SPI_SIMPLE |  |
| --- | --- |
| Belongs to | TC_F_COMMUNICATION_SPI |
| Description | Test for SPI communication. Doesn’t have code. It’s indirectly passed by using any of the RDAC tests |
| Required | MOD5213 connected to a single SlotCard with I2C bus operational. |
| Input | “[1-5]” – Keyboard shortcut from the Test Menu |
| Output | The result of SPI communication, saved in the I2CtoSPIBridge buffer PASSED: the “receive Buffer” is not empty. NOT PASSED: Empty always. |
| 24/03/15 | SUCCESSFUL |